Performance Analysis of YOLOv5 Nano on Zynq UltraScale+ ZCU102

1. System Architecture

The system follows a Heterogeneous System-on-Chip (SoC) Architecture, leveraging the Zynq UltraScale+ MPSoC to create a high-performance hardware-accelerated pipeline. The architecture is physically and logically divided into two main domains.

1.1 Hardware Domain: The Zynq UltraScale+ Platform

- Processing System (PS): * Application Processing Unit (APU): Quad-core ARM Cortex-A53 running at 1.2 GHz. It hosts the Linux OS and executes the Python application logic, image pre/post-processing, and system management.

  - Memory Controller: Manages the 4GB DDR4 SODIMM, which acts as a shared unified memory space for both the CPU and the FPGA.

- Programmable Logic (PL):

  - Deep Learning Processor Unit (DPU): The B4096 IP core is instantiated here. It is a specialized multi-threaded inference engine optimized for CNNs.

  - Interconnect: High-performance AXI4 interfaces connect the DPU to the PS memory controller, enabling the DPU to read image tensors and write feature maps with ultra-low latency.

1.2 Software Architecture & Execution Flow

The software stack uses a layered approach to bridge the YOLOv5 model to the hardware:

- Application Layer: A Python script utilizing the VART (Vitis AI Runtime) API.

- Runtime Layer: XRT (Xilinx Runtime) manages the hardware buffers and schedules tasks to the DPU.

- Kernel Layer: The compiled .xmodel contains the instruction set and weight data that configure the DPU's execution for the YOLOv5 Nano graph.

1.3 End-to-End Data Pipeline

- Input: The ARM CPU captures a $1920 \times 1080$ frame from the USB webcam via the V4L2 driver.

- Pre-process (PS): OpenCV resizes the frame to $640 \times 640$ and normalizes data to INT8.

- Offload: The ARM CPU triggers the DPU via an interrupt and points it to the memory address of the input tensor.

- Hardware Acceleration (PL): The DPU executes the 4.49 GOP backbone in 1.99 ms.

- Post-process (PS): The ARM CPU reads the resulting tensors, applies the detection head scaling (Subgraphs 2-4), and executes Non-Maximum Suppression (NMS) to generate final object coordinates.

2. Introduction

This document presents the deployment and performance evaluation of the YOLOv5 Nano object detection model on the AMD-Xilinx ZCU102 FPGA platform. The implementation utilizes a quantized INT8 .xmodel executed on the DPUCZDX8G hardware accelerator core.

3. Design Partitioning Analysis

The model was compiled for the B4096 DPU architecture. A functional split was executed between the FPGA’s Programmable Logic (PL) and the ARM CPU’s Processing System (PS) to optimize execution.

3.1 Architectural Mapping (Terminal Validation)

Analysis via xdputil xmodel -l confirms a 5-subgraph partition:

- Subgraph 1 (DPU): Encapsulates the bulk of the neural network (4.49 Billion Operations). It processes a $640 \times 640$ input and outputs three detection scales ($20 \times 20$, $40 \times 40$, $80 \times 80$).

- Subgraphs 2–4 (CPU): These "Heads" handle the final coordinate and class scaling.

- User (CPU): Handles initial Quantization Stubs and final Non-Maximum Suppression (NMS).

4. Performance Metrics

4.1 Hardware Throughput & Latency

Raw hardware performance was measured using a multi-threaded benchmark to determine the peak capability of the FPGA fabric.

| Metric | Measured Value |
| --- | --- |
| Total Frames Processed | 30,029 |
| Test Duration | 60.00 seconds |
| Hardware Throughput | 500.46 FPS |
| Average DPU Latency | 1.99 ms |

4.2 Resource Utilization (DPUCZDX8G_B4096)

The ZCU102 utilizes the following estimated hardware resources for the B4096 DPU core configuration:

- Workload: 4,488,286,000 GOP per image.

- Memory Footprint: 9.8 MB Workspace (REG_1) + 2.1 MB Constants (REG_0).

- Resource Intensity: High usage of DSP48 slices (approx. 700–800) and BRAM blocks to maintain the 500 FPS throughput.

5. Power Consumption and Efficiency

Power measurements were taken using the onboard TI INA226 sensors to evaluate the energy cost of high-speed inference.

| State | Power Consumption (W) |
| --- | --- |
| Idle Board Power | 10.50 W |
| Active Inference Power | 14.22 W |
| Net DPU Power (Dynamic) | 3.72 W |
| Energy Efficiency | 134.5 FPS/Watt |

6. Systematic Bottleneck Analysis

A significant discrepancy exists between the Hardware Capability (500 FPS) and the Application Performance (approx. 30 FPS) when running the yolov5_nano_webcam.py script.

- I/O Bottleneck: The USB webcam is capped at 30 FPS.

- Software Overhead: Python-based pre-processing (OpenCV resize) and post-processing (NMS) on the ARM CPU introduce significant latency that prevents the DPU from reaching its 500 FPS potential.

- Partitioning Impact: The exit from FPGA to CPU for subgraphs 2–4 introduces memory copy overhead that is negligible at 30 FPS but would become critical at higher throughputs.

7. Conclusion

The YOLOv5 Nano implementation on the ZCU102 achieves a state-of-the-art hardware latency of 1.99 ms. While the DPU is capable of exceeding 500 FPS, system-level performance is currently limited by the software pipeline. Future optimizations should focus on C++ implementation of NMS and multi-stream batching to fully utilize the DPU capacity.